# Problem Statement

Design and implement a secure, scalable two-site network (Branch + HQ) that supports VLANs with inter-VLAN routing, OSPFv2, static routing to the Internet, DHCP (server + relay), WLAN, NAT (static + PAT), and ACLs. The design uses VLSM across 192.168.0.0/24 (Branch) and 172.10.0.0/24 (HQ), minimizes address waste, and passes full end-to-end connectivity tests without screenshots (configs are visible in the .pkt file).

# Design and Implementation

## Addressing and Gateways

| Device Name |  | Interface/Default Gateway | IP address/prefix |
| --- | --- | --- | --- |
| Internet |  | G0/0 | 138.25.88.85/30 |
|  |  | S0/1/1 | 209.165.200.1/30 |
| R1 [[R1Na me] [[R1Na me]]G0/ 0 |  | F0/0 | 192.168/0/113/29 (to Sw4) |
|  |  | S0/0/0 | 172.10.0.145/30 (to R4) |
|  |  | S0/0/1 | 209.165.200.2/30 (to internet) |
| R2 | [[R2Na me]] | F0/1.10 | 192.168.0.65/27 (vlan 10) |
|  |  | F0/1.20 | 192.168.0.1/26 (vlan 20) |
|  |  | F0/1.99 | 192.168.0.97/29 (vlan 99) |
|  |  | F0/0 | 192.168.0.115/29 (to Sw4) |
| R3 | [[R3Na me]] | F0/0 | 192.168.0.114/29 (to sw4) |
|  |  | F0/1 | 192.168/0/105/29 (to Sw2) |

| Device Name | Interface/Default Gateway | IP address/prefix |
| --- | --- | --- |
| [[R4Nam [[R4Name]] [[R4Name]] | S0/0/1 | 172.10.0.149/30 (to R5) |
|  | S0/1/0 | 172.10.0.157/30 (to r6) |
|  | S0/0/0 | 172.10.0.146/30 (to R1) |
| R5 [[R5Name]] [[R5Name]] | F0/0 | 172.10.0.129/28 (to Sw3) |
|  | S0/0/0 | 172.10.0.150/30 (to R4) |
|  | S0/0/1 | 172.10.0.153/30 (to R6) |
| R6 [[R6Name]] [[R6Name]] [[R6Name]] | F0/0 | 172.10.0.1/25 (to Sw5) |
|  | F0/1 | 172.10.1.1/24 (to WR) |
|  | S0/0/0 | 172.10.0.158/30 (to R4) |
|  | S0/0/1 | 172.10.0.154/30 (to R5) |
| Sw1 | VLAN 99 | 192.168.168.0.98 |
|  | Default Gateway | 192.168.0.97 |
| Host A1 | NIC VLAN 10 | DHCP |
| Host A2 | NIC VLAN 20 | 192.168.0.2 |
|  | Default Gateway | 192.168.0.1 |
| Host B | NIC | 192.168.0.106 |
|  | Default Gateway | 192.168.0.105 |
| Host C | NIC | DHCP |
| Host D | NIC | 172.10.0.2 |
|  | Default Gateway | 172.10.0.1 |
| Mobile Host | Wireless NIC | DHCP |
| External Host | NIC | 138.25.88.86/30 |

## VLANs & Inter-VLAN Routing (Branch)

- Sw1: VLAN 10 (students), VLAN 20 (staff), VLAN 99 (management/native). Trunk to R2 allows {10,20,99} with native 99.

- R2: Subinterfaces F0/1.10, .20, .99 with encapsulation dot1q <VLAN> and the configured gateway IPs (router-on-a-stick).

## Routing

- OSPFv2: Process 10 on all routers; Area 0 end-to-end. Router IDs set; R1 given highest OSPF interface priority on the multi-access network; passive on user VLANs. Default route configured on R1 to Internet and advertised with default-information originate.

- Edge static routes: Internet↔R1 next-hop routes so public and translated addresses are reachable.

## NAT

- Static NAT: HQ Host D mapped to its assigned public IP for inbound reachability from the External Host.

- Dynamic PAT: ACL 1 matches Branch VLAN 10 & 20; PAT pool configured; overload enabled.

## ACLs

- VTY-BLOCK (named standard): Deny all external sources to R3 VTY; permit only Branch LAN-B.

- ACL 10 (numbered standard): Deny HQ LAN-D to HQ LAN-C; otherwise permit.

## DHCP

- R4 as DHCP server with excluded first five addresses per pool.

- DHCP Relay: On R2 (VLAN 10) and on the HQ interface for LAN-C to forward to R4.

## WLAN

- WR Internet Port: Static IP as designed.

- DHCP: 192.168.1.0/24 scope, first five excluded; GW = first usable; max users 100.

- Wireless: SSID LANE (2.4 GHz), WPA2-AES, passphrase cisco. Mobile host joins and receives DHCP.

# Troubleshooting

## 1. OSPF neighbors stuck at 2-Way on multi-access

Cause: DR/priority mismatch and a passive interface left enabled.

Fix: Set R1 to highest OSPF priority, removed passive on the segment, reset OSPF—neighbors reached FULL.

## 2. No Internet from Branch after enabling PAT

Cause: Missing return route for PAT pool on Internet router and NAT inside/outside not applied on R1.

Fix: Added static route for pool; corrected ip nat inside/outside on R1; verified with show ip nat translations.

3. External Host cannot reach Host D via static NAT  Cause: Edge ACL blocked inbound; missing route to LAN-D.

Fix: Permitted the public IP; ensured OSPF route to LAN-D; inbound tests succeeded.

## 4. VLAN 10 clients not getting DHCP

Cause: No helper on R2 subinterface; trunk native VLAN mismatch.

Fix: Added ip helper-address to F0/1.10; set trunk native to 99 on both ends; leases issued.

## 5. WLAN ‘Internet port down’

Cause: Internet port not addressed and peer interface shut.

Fix: Assigned static IP/GW to WR Internet port; no shut on the R6 peer; mobile host gained Internet.

## 6. Inter-VLAN routing intermittent

Cause: Access ports on Sw1 assigned to wrong VLANs.

Fix: Audited and corrected access VLANs; verified with show vlan brief.

## 7. No communication on point-to-point serial link

Cause: Did not add clock rate to the DTE side of the link.

Fix: Added clock rate of 6400 on the DTE side of the link.

## Final Validation

- Branch VLAN 10/20, Branch LAN-B, HQ LAN-C/D, and WLAN can all reach the

Internet.

- External Host reaches Host D via the configured static NAT.

- OSPF routes present and stable; default route learned network-wide.

- ACLs enforce the required restrictions.

- All deliverables pass testing.